UART Communication and Data Integrity UART Communication and Data Integrity

| Module | ET4047 |
| --- | --- |
| Date | 04/10/2026 |
| Lab No. | Lab 5 |
| Author | Luke Griffin |

# 1  Setup

## Equipment

| Item | Qty | Notes |
| --- | --- | --- |
| Arduino Nano (classic) or Nano Every, on Uno-style breakout shield | 2 | All the code works on either board. Task 1 detects the board automatically. |
| 16×2 LCD with I2C backpack, or Adafruit RGB LCD shield | 1–2 | Either works: there is a folder of code for each display. |
| Jumper wires (male–male / male–female) | ~10 |  |
| USB cables | 2 | One per board. |
| Push button | 1 | Task 4 only. |

## Software

- Arduino IDE. Select the board you have: Tools → Board → Arduino AVR Boards → Arduino Nano (classic), or Arduino megaAVR Boards → Arduino Nano Every (install the megaAVR package from the Boards Manager first). Classic Nano clones may also need Tools → Processor → ATmega328P (Old Bootloader).

- The library for your display only: LiquidCrystal I2C (by Frank de Brabander) for the I2C backpack LCD, or Adafruit RGB LCD Shield Library (by Adafruit) for the RGB shield. Tools → Manage Libraries… and search for it.

- SoftwareSerial is built in. There is nothing to install.

- The lab code, W05_UART_Lab_Code.zip. Unzip it somewhere you can find it again.

## Choose the code for your display

The zip has one folder for each type of display. Use only the folder for your display. Every sketch is a single .ino file that opens with File → Open, and the comments in it explain each step.

| Display | How to recognise it | Folder to use |
| --- | --- | --- |
| 16×2 LCD with I2C backpack | A separate screen with a small board on the back and four wires | I2C_Backpack_LCD |
| Adafruit RGB LCD shield | Plugs straight onto the breakout board and has five buttons | Adafruit_RGB_LCD_Shield |

## LCD wiring (all tasks)

I2C LCD connections

| From |  | To | Purpose |
| --- | --- | --- | --- |
| LCD GND | → | Nano GND | Ground |
| LCD VCC | → | Nano 5V | Power |
| LCD SDA | → | Nano A4 | I2C data |
| LCD SCL | → | Nano A5 | I2C clock |

Figure 1: LCD wiring. The pinout is the same on the classic Nano and the Nano Every.

Using the Adafruit RGB LCD shield instead? Plug it onto the Uno-style breakout shield. It uses the same I2C pins (SDA on A4, SCL on A5), so no wiring is needed.

# 2  Task 1: UART loopback on a single board

Objective: connect the board’s UART output straight back into its own input. Everything the board transmits should arrive back on its receive pin, which proves that the UART, the wiring and your code are working before you involve a second board.

## Wiring

Loopback connection (plus the LCD wiring from Section 1)

| From |  | To | Purpose |
| --- | --- | --- | --- |
| Nano D1 (TX) | → | Nano D0 (RX) | Loopback: output feeds straight into input |

Figure 2: Task 1 wiring: one jumper from D1 (TX) to D0 (RX), which are next to each other on the header

| Figure 3: Loopback jumper and LCD wiring | Figure 4: LCD without line wrapping |
| --- | --- |

|  | Classic Nano (ATmega328P) | Nano Every (ATmega4809) |
| --- | --- | --- |
| Hardware UARTs on the pins | One, Serial. It is shared with the USB link. | A spare one, Serial1. USB has its own channel. |
| Port used by the Task 1 code | Serial | Serial1 |
| Loopback jumper while uploading | Remove it, or the upload fails | Can stay in place |
| Serial Monitor during Task 1 | Shows the messages being sent. Don’t type in it: typed characters also arrive on D0. | Stays empty |

Why the classic Nano behaves differently On the classic Nano, D0 and D1 are also wired to the USB-to-serial chip. Anything the board sends on D1 goes both to the jumper and to your PC, and the upload uses the same pins, so the jumper gets in the way while uploading.

## What happens

Code Upload Task1_Loopback from your display’s folder. On a classic Nano, take the jumper off while uploading.

Once a second the board sends a word (Hello, Test or Loop) out of its TX pin, D1. The jumper carries it straight back into its RX pin, D0, and every character that arrives is shown on the LCD. When a row is full the text moves down to the next row; when both rows are full the screen clears and starts again at the top.

Expected result: the display fills row by row with HelloTestLoopHel / loTestLoopHelloT, clears, and starts again at the top.

## Task 1 questions

### 1a  Run it

Upload the code, wire the jumper and check the messages appear. (record) Take a photo of the LCD.

### 1b  Break the loop

While the sketch is running, pull the D1–D0 jumper out, wait a few seconds, then put it back. What happens on the LCD, and why? Is anything lost while the wire is out?

Your answer:

### 1c  Without the wrapping

In the receive loop, temporarily replace lcdPutChar(c); with lcd.write(c); and run it again. Describe what you see (compare with Figure 4), then put lcdPutChar() back. Why does the LCD not wrap on its own?

Your answer:

### 1d  Timing

Each character is sent as a 10-bit frame (1 start bit, 8 data bits, 1 stop bit), and at 9600 baud each bit lasts 1/9600 s. How long does it take to transmit the word Hello at 9600 baud? How many characters per second can a 9600-baud link carry at most?

Your answer:

# 3  Task 2: Communication between two boards

Objective: Board A (the sender) transmits a numbered message every second; Board B (the receiver) collects the characters into a complete line and shows it on its LCD and Serial Monitor.

## Wiring

Label your boards A (sender) and B (receiver). The LCD connects to Board B only.

Board-to-board connections (the TX/RX wires cross over)

| From |  | To | Purpose |
| --- | --- | --- | --- |
| A D11 (TX) | → | B D10 (RX) | A’s data travels to B |
| A D10 (RX) | → | B D11 (TX) | Return path (not used this week) |
| A GND | → | B GND | Common reference voltage. Essential. |

Figure 5: Task 2 wiring: TX goes to RX in both directions, grounds joined, LCD on Board B

Figure 6: The two-board set-up in the lab

Connect both boards to the PC by USB. Each gets its own COM port. Upload the sender code to A and the receiver code to B, selecting the correct port each time. Open a Serial Monitor for each board, both at 9600 baud. (In the IDE, open a second window with File → New to get a second monitor.)

## What happens

Code Upload Task2_Sender to Board A and Task2_Receiver to Board B, both from your display’s folder.

Every second Board A sends a numbered message, such as Hello 12, with a newline character on the end. UART only delivers one character at a time, so Board B saves each character in a buffer (an array) until the newline arrives. Then it knows the message is complete and shows it on its LCD and Serial Monitor.

## Task 2 questions

### 2a  Run it

Upload both sketches and run them. (record) Paste a few lines from both Serial Monitors. They should look like this:

Board A Serial Monitor

Sender ready Sent: Hello 0 Sent: Test 1 Sent: Loop 2

Board B Serial Monitor

Received: Hello 0 Received: Test 1 Received: Loop 2

### 2b  Break it on purpose

Make each change below one at a time, observe what Board B shows, then undo it. (record) Fill in the table and explain each result. Think about the baud rate (both boards must use the same one), which pin drives which (TX must go to RX), and the shared ground.

| Change | What Board B shows | Why |
| --- | --- | --- |
| Remove the GND–GND wire |  |  |
| Change the receiver to link.begin(4800) |  |  |
| Connect A’s TX to B’s TX instead of RX |  |  |
| Set MAX_LEN to 4 on the receiver |  |  |

Note A missing ground sometimes appears to work, because the boards may share a ground through the PC’s USB ports. It is still wrong, and it fails as soon as one board runs from a battery.

### 2c  Questions

- Why does the receiver need the '\n' at the end of each frame? What would B display if the sender left it out?

- What would happen if the len < MAX_LEN check were removed and A sent a 100-character message?

Your answer:

# 4  Task 3: Secret messages with a Caesar cipher

Objective: anyone who connects to the wire can read what Board A sends. In this task Board A scrambles (encrypts) each message with a Caesar cipher before sending it, and Board B unscrambles (decrypts) it. The wiring is the same as Task 2.

## The Caesar cipher

Every letter is replaced by the letter a fixed number of places further along the alphabet. That number is the key, also called the shift. With a shift of 3, A becomes D, B becomes E, and so on. At the end the alphabet wraps round: X becomes A, Y becomes B and Z becomes C. To decrypt, move each letter back by the same number. Spaces and digits are not changed.

| Message (plain text) | Sent on the wire (shift 3) |
| --- | --- |
| Hello | Khoor |
| Embedded | Hpehgghg |
| Software | Vriwzduh |

## What happens

Code Upload Task3_Sender to Board A and Task3_Receiver to Board B, both from your display’s folder.

Every 2 seconds Board A encrypts the next message with a shift of 3 and sends the scrambled text. Board B shows the scrambled text it received on the top row of the LCD and the decrypted message on the bottom row. Both Serial Monitors show the message before and after encryption.

## Task 3 questions

### 3a  Run it

Upload both sketches and run them. (record) Paste a few lines from both Serial Monitors. They should look like this:

Board A Serial Monitor

Sender ready Plain: Hello Sent: Khoor Plain: Embedded Sent: Hpehgghg

Board B Serial Monitor

Received: Khoor Decrypted: Hello Received: Hpehgghg Decrypted: Embedded

### 3b  The wrong key

Change SHIFT to 4 in the receiver only and upload it again. What does Board B show for Hello? Why must both boards use the same key? Put it back to 3 afterwards.

Your answer:

### 3c  By hand

1. Encrypt the word Zebra with a shift of 3. Which letter has to wrap round to the start of the alphabet?

2. Decrypt Pdwkv (shift 3).

Your answer:

### 3d  Is it secure?

1. How many different keys can a Caesar cipher have?

2. Someone listening on the wire sees Wkh grru frgh lv 1234. How could they read it without knowing the key?

3. Why is a Caesar cipher not used to protect real data?

Your answer:

# 5  Task 4: Checking messages with SHA-1

Objective: noise or a loose wire can change bits on the way, and UART does not notice. In this task Board A sends a hash with every message, and Board B uses it to check whether the message arrived unchanged. The wiring is the same as Task 2, plus a push button on Board A that damages a message on purpose.

Figure 7: Task 4 wiring: the Task 2 link, plus a push button between Board A D2 and GND

## What is a hash?

A hash function turns any message into a short, fixed-size “fingerprint”. SHA-1 always gives 40 hexadecimal characters. The same message always gives the same hash, but changing even one letter gives a completely different one:

| Message | SHA-1 hash |
| --- | --- |
| Testing | 0820b32b206b7352858e8903a838ed14319acdfd |
| Tasting | ea1e43d415e84b553d0625a9ac75af4ea2a27fe9 |

So if Board B works out the hash of the message it received and gets the same answer as Board A, the message is almost certainly unchanged. If the two hashes are different, something was damaged on the way.

What Board A sends

| Testing | ¦ | 0820b32b…319acdfd | \n |
| --- | --- | --- | --- |
| message (any length) | separator | SHA-1 hash: 40 hex characters | end of frame |

## What happens

Code Upload Task4_Sender to Board A and Task4_Receiver to Board B, both from your display’s folder. The SHA-1 code is already at the bottom of both sketches: you do not need to change it.

Every 2 seconds Board A works out the hash of the next message and sends the message and the hash together. Board B works out the hash of the message it received and compares the two. The LCD shows the message on the top row, and OK or FAIL with a running count (e.g. OK   3/4) on the bottom row. While you hold the button on Board A, it changes one bit of the message before sending it, but still sends the hash of the original, just like a message damaged on the wire.

## Task 4 questions

### 4a  Run it

Upload both sketches and run them. (record) Paste one result from Board B’s Serial Monitor. Every message should pass:

Board B Serial Monitor

Message: Testing Received: 0820b32b206b7352858e8903a838ed14319acdfd Computed: 0820b32b206b7352858e8903a838ed14319acdfd Result: OK

### 4b  Damage a message

Hold the button on Board A while a message is sent. (record) Paste the FAIL result from Board B and the matching line from Board A. They should look like this:

Board A Serial Monitor (button held)

Sent (TAMPERED): Dmbedded¦0fd17b94c9576216ee0a47001e3fdbb1920a1f50

Board B Serial Monitor

Message: Dmbedded Received: 0fd17b94c9576216ee0a47001e3fdbb1920a1f50 Computed: 106b572c38133fff8835b5165f4d092b9042db0e Result: FAIL

Only one bit of one letter changed (E became D). Roughly how many of the 40 hash characters are different?

Your answer:

### 4c  Questions

1. Can Board B fix a damaged message using the hash, or only tell that it is damaged?

2. If the damage hits the hash instead of the message, what does Board B report? Is that the right result?

3. Testing is 7 characters, but the line sent is 49 characters long. Why might a smaller check be better on a slow link?

Your answer: